# Lab 4c: Raster Zonal Statistics for Heat Vulnerability

Intro GIS for Public Health · ArcGIS Pro · Day 4

## Public health question

Summer heat is the leading weather-related cause of death in the United States, but the burden falls unevenly across neighborhoods. Which Philadelphia census tracts have the highest land-surface temperatures during summer, and do those tracts also carry the highest social vulnerability? Identifying that overlap is an early step in targeting heat-health interventions — cooling centers, tree-planting programs, and outreach to isolated elderly residents.

## What you will learn

- Enable the Spatial Analyst extension in ArcGIS Pro

- Add and symbolize a single-band raster using a classified color ramp

- Reclassify a raster into meaningful categories using the Reclassify tool

- Write a simple map algebra expression in the Raster Calculator

- Run Zonal Statistics as Table to summarize raster values by polygon zones

- Join a statistics table back to a feature class and symbolize the result

- Compare two thematic layers (LST and SVI) to identify doubly-burdened tracts

## What you will produce

- A choropleth map of census tracts symbolized by zonal mean land-surface temperature (LST)

- A side-by-side or inset comparison with SVI quartile symbology showing spatial overlap

- (Bonus) A bivariate choropleth combining LST and SVI into a single map

- A short written interpretation (~100 words) identifying the highest heat-vulnerability tracts and the limits of this analysis

## Data

All data are on the W: drive. Throughout this lab, “W:” refers to the network or local storage drive where your instructor has placed the lab files — for example, W:\Lab04c\data\. Adjust the path to match your setup.

| Layer | File name | Description | Source |
| --- | --- | --- | --- |
| Summer LST raster | phl_LST_summer.tif | Single-band Landsat 8/9 summer land-surface temperature, Philadelphia extent, units °C, already processed | Instructor-provided (USGS Landsat / NOAA HEAT.gov) |
| Philadelphia census tracts with SVI | phl_tracts_SVI.shp | 2020 TIGER tract boundaries with CDC/ATSDR SVI 2022 overall percentile field (RPL_THEMES) pre-joined | Instructor-provided (CDC/ATSDR SVI 2022) |

Note: Both layers are projected to NAD83 PA State Plane South (EPSG:2272, US feet). The Spatial Analyst extension is required for this lab. Confirm with your instructor that it is activated on your license.

## Before you begin

Prerequisites: You should be comfortable adding data, using the Contents pane, and opening geoprocessing tools (covered in Labs 1–2). No prior raster experience is required — this lab introduces it.

Time estimate: 90–110 minutes.

Have open: ArcGIS Pro with a new project saved to your W: drive folder for Lab 4c. The default file geodatabase created with the project will store your output rasters and tables.

## Part 1: Enable Spatial Analyst and Add the LST Raster

### Enable the Spatial Analyst extension

- On the Project tab, click Licensing. Check the box next to Spatial Analyst and click OK. If the box is grayed out, your license does not include it — ask your instructor.

### Create a project and add data

- On the Insert tab, click New Map.

- In the Catalog pane (right side), navigate to your W: drive folder. Right-click phl_LST_summer.tif and choose Add to Current Map. Repeat for phl_tracts_SVI.shp.

- In the Contents pane (left side), drag phl_tracts_SVI above phl_LST_summer.tif so the tract boundaries appear on top.

### Inspect the LST raster

- Click phl_LST_summer.tif in the Contents pane to select it. On the Raster Layer tab, click Identify, then click any pixel to see its LST value in °C.

- Right-click the layer, choose Properties > Source, and note the cell size, coordinate system, and value range. Close the dialog.

[Figure: Contents pane showing LST raster and tracts layer; Identify pop-up showing a pixel value in °C]

### Symbolize the LST raster with a heat color ramp

- In the Contents pane, double-click the color swatch under phl_LST_summer.tif to open the Symbology pane.

- Under Primary symbology, change the method from Stretch to Classify.

- Set Classes to 5 and Classification method to Natural Breaks (Jenks).

- Under Color scheme, choose a sequential ramp from light yellow/white to deep red or orange (e.g., “Yellow to Red”). Click Apply.

[Figure: Symbology pane with Classify method, 5 classes, yellow-to-red color ramp applied to LST raster]

Public health note: Land-surface temperature (LST) measured by satellite is a surface thermal emission, not air temperature and not personal heat exposure. LST correlates with urban heat island patterns — impervious surfaces, dark rooftops, and sparse tree cover drive high values. It is a useful spatial indicator for prioritizing areas but should not be interpreted as a measure of what residents actually experience.

## Part 2: Reclassify LST into Temperature Classes

Reclassification converts a continuous raster into discrete integer categories — useful for thematic mapping and side-by-side comparison with SVI quartiles.

- On the Analysis tab, click Tools. Search for “Reclassify.” Under Spatial Analyst Tools > Reclass, open Reclassify.

- Set parameters:

| Parameter | Value |
| --- | --- |
| Input raster | phl_LST_summer.tif |
| Reclass field | Value |
| Output raster | lst_reclass (saved to your project geodatabase) |

- In the Reclassification table, click Classify. Set the Method to Natural Breaks (Jenks) and Classes to 5. Click OK. The table will populate with break values.

- In the New column, assign values 1 through 5 (1 = coolest class, 5 = hottest class). Confirm the NoData row maps to NoData.

- Click Run.

- When complete, lst_reclass is added to the map. Open its Symbology pane and apply a five-class sequential ramp (light to dark, matching your LST raster) so the classes are visually consistent.

[Figure: Reclassify tool dialog with break values and New values 1–5 filled in]

Note: The reclassified raster stores integer class codes (1–5), not temperature values. If you hover over a cell with Identify, you will see 1, 2, 3, 4, or 5 — not degrees Celsius. Keep the original phl_LST_summer.tif for the zonal statistics step; that tool needs the continuous values.

## Part 3: (Optional) Raster Calculator — Unit Conversion or Simple Index

This step is optional. Skip to Part 4 if your instructor directs you to.

- On the Analysis tab, click Tools. Search for “Raster Calculator.” Under Spatial Analyst Tools > Map Algebra, open Raster Calculator.

- To convert LST from Celsius to Fahrenheit, click phl_LST_summer.tif in the Rasters panel (this inserts it with quotes), then type the rest of the expression:

- ("phl_LST_summer.tif" * 9 / 5) + 32

- Name the output lst_fahrenheit. Click Run. Use Identify on a pixel to confirm values are ~32–50 degrees higher than the Celsius raster.

Warning: The Raster Calculator is a strict parser. Layer names must be quoted and spelled exactly as shown in the Rasters panel. Do not add spaces inside function names (Con( not Con (). Use the panel buttons to insert operators whenever possible.

[Figure: Raster Calculator dialog with Fahrenheit conversion expression filled in]

## Part 4: Zonal Statistics as Table

Zonal Statistics as Table summarizes raster pixel values within each polygon zone (census tracts here) and writes count, mean, min, max, and more to a standalone table. You then join that table to your tracts to map the result.

Public health note: Zonal statistics is the single most useful raster operation for public health spatial analysis. The same workflow applies to any continuous raster: zonal mean PM2.5 by census tract, zonal mean NDVI (tree canopy) by neighborhood, zonal mean impervious surface by block group. Learn this once and you can apply it to dozens of environmental justice questions.

### Run the tool

- On the Analysis tab, click Tools. Search for “Zonal Statistics as Table.” Under Spatial Analyst Tools > Zonal, open Zonal Statistics as Table.

- Set parameters:

| Parameter | Value |
| --- | --- |
| Input raster or feature zone data | phl_tracts_SVI |
| Zone field | GEOID |
| Input value raster | phl_LST_summer.tif |
| Output table | lst_zonal_stats (to your project geodatabase) |
| Statistics type | MEAN (optionally also MIN and MAX) |

- Click Run (30–60 seconds). Open lst_zonal_stats from the Catalog pane. Confirm one row per tract and plausible °C values in the MEAN column.

[Figure: Zonal Statistics as Table dialog filled in; output table showing GEOID, COUNT, MEAN columns]

Note: GEOID is the standard 11-digit census tract FIPS code (e.g., 42101000100). It must appear identically in both the shapefile and the output table to join correctly. Open the tract attribute table to confirm the field name if unsure.

### Join the table to the tracts

- In the Contents pane, right-click phl_tracts_SVI and choose Joins and Relates > Add Join.

- Set join parameters:

| Parameter | Value |
| --- | --- |
| Input Join Field | GEOID |
| Join Table | lst_zonal_stats |
| Join Table Field | GEOID |
| Keep all target features | Checked |

- Click OK. The join is temporary (session only, not written to the shapefile). Open the Attribute Table and scroll right to confirm lst_zonal_stats.MEAN is present and populated.

[Figure: Attribute table of phl_tracts_SVI showing joined MEAN field]

Warning: If many tracts show <Null> in the MEAN column, the field types likely do not match — one is a string, the other an integer. Cancel the join, confirm GEOID is a string (text) in both tables, and re-join. Ask your instructor if the mismatch persists.

### Symbolize tracts by mean LST

- Double-click the color swatch under phl_tracts_SVI to open Symbology. Set Primary symbology to Graduated Colors, Field to lst_zonal_stats.MEAN, Classes to 5, Method to Natural Breaks (Jenks). Apply the same yellow-to-red ramp used for the raw raster. Click Apply.

Tracts are now colored by average summer LST. Hottest tracts appear in dark red.

[Figure: Philadelphia census tract map, graduated-color symbology by zonal mean LST, hottest tracts in dark red]

## Part 5: Classify SVI and Build the Comparison Display

### Classify SVI into quartiles

The SVI overall percentile field RPL_THEMES runs from 0 to 1, where values near 1 indicate the highest social vulnerability. Quartiles divide tracts into four equal groups (Q1 = least vulnerable, Q4 = most vulnerable).

- On the Insert tab, click New Map to add a second map frame. Add phl_tracts_SVI to this second map.

- In the second map’s Symbology pane, set Primary symbology to Graduated Colors, Field to RPL_THEMES, Classes to 4, Method to Quantile. Apply a light-to-dark purple ramp (dark = Q4, most vulnerable). Click Apply.

[Figure: Second map frame showing tracts by SVI quartile, dark purple = highest vulnerability]

### Identify doubly-burdened tracts

- Arrange the two map frames side by side. On the View tab, use Link Views to synchronize pan and zoom.

- Visually identify tracts that are dark red on the LST map AND dark purple on the SVI map. These are doubly-burdened tracts — highest heat and highest vulnerability.

- Use the Identify tool to click on those tracts and record their GEOIDs, mean LST, and SVI percentile.

Public health note: Spatial co-occurrence of high LST and high SVI is not evidence of worse health outcomes. LST is a surface thermal proxy, not air temperature, personal exposure, or illness. A full heat vulnerability assessment would add emergency department visits for heat illness, air-conditioning access, cooling center proximity, and age structure. This map is a prioritization tool, not a conclusion.

### (Bonus) Bivariate choropleth

ArcGIS Pro does not have a built-in bivariate renderer. You will approximate one manually.

- Right-click phl_tracts_SVI, open Attribute Table, and click Add Field. Name it BVCLASS, type Text, length 4. Save.

- Click Calculate Field. Using Python, write an expression that assigns a two-character code combining the LST class (1–5) and SVI quartile (1–4) — for example, "5Q4" for a tract in the hottest LST class and highest SVI quartile. Your instructor will supply the break values.

- Symbolize by Unique Values on BVCLASS. Assign a bivariate color grid: the darkest, most alarming color in the upper-right corner (highest LST + highest SVI).

[Figure: Bivariate choropleth of Philadelphia tracts; darkest cells indicate heat-vulnerability overlap]

## Deliverables

- Map 1 — Zonal Mean LST by Census Tract. Fully labeled layout (title, legend, north arrow, scale bar, data sources, your name). Tracts by lst_zonal_stats.MEAN, yellow-to-red graduated color, 5 Natural Breaks classes.

- Map 2 — SVI Quartile by Census Tract. Same extent and scale as Map 1. Tracts by RPL_THEMES, 4 quantile classes, light-to-dark purple.

- Written interpretation (~100 words). Identify 2–3 tracts (by GEOID or neighborhood) in both the hottest LST class and SVI Q4. Describe the pattern. State what the analysis cannot show — specifically that LST is a surface proxy, not air temperature or health outcome, and that no morbidity data are present.

- (Bonus) Map 3 — Bivariate Choropleth (LST × SVI). Same layout requirements; include a bivariate legend.

Export all maps as PDF. Label any AI-assisted output clearly.

## Using AI in this lab

### Suggested task: interpret the comparison map

After completing Maps 1 and 2, paste the following prompt into Claude (claude.ai) or another AI assistant. Do not include any identifying data, tract-level health records, or PHI.

“I have created two maps of Philadelphia census tracts: one showing summer land-surface temperature (LST) as a heat proxy, the other showing CDC/ATSDR Social Vulnerability Index (SVI) quartiles. High-LST, high-SVI tracts appear concentrated in [describe the pattern you observed — e.g., ‘North Philadelphia and Kensington’]. Draft a 200-word equity-framed summary suitable for a neighborhood advisory board presentation.”

Critical review task. Read the AI draft and flag every sentence where the AI: - Describes LST as heat exposure or predicts illness (LST is a surface proxy, not personal exposure) - Implies a health outcome (emergency visits, deaths) absent from this dataset - Makes causal claims not supported by the two layers you analyzed

Revise the draft to correct those overstatements. Include two sentences in your deliverable explaining what you changed and why.

### AI guardrails (always apply)

- AI assists with scaffolding (draft text, example expressions), explaining (tool mechanics, concepts), and critiquing (your map layout or interpretation).

- AI does not make analytic decisions, choose datasets, set classification breaks, or interpret maps it cannot see.

- Never paste PHI or identifiable data into a public AI system.

- Label all AI-assisted output: for example, “AI-assisted draft, revised by [your name].”

## QGIS equivalent

| ArcGIS Pro operation | QGIS equivalent |
| --- | --- |
| Enable Spatial Analyst | No extension needed; most tools are built in via GDAL/SAGA in the Processing Toolbox |
| Add and display raster | Layer > Add Layer > Add Raster Layer |
| Raster symbology (classified, color ramp) | Layer Properties > Symbology > Singleband pseudocolor; choose a color ramp and set classification mode to Natural Breaks |
| Reclassify tool | Processing Toolbox > SAGA > Raster tools > Reclassify grid values; or Raster Calculator with conditional expressions |
| Raster Calculator | Raster > Raster Calculator |
| Zonal Statistics as Table | Processing Toolbox > Zonal statistics (outputs statistics as new fields on the zone polygon layer); or SAGA > Zonal grid statistics for a separate table |
| Add Join | Layer Properties > Joins > Add Join (same concept, same dialog structure) |

QGIS with the default SAGA and GDAL plugins handles all operations in this lab without a paid extension. The workflow is nearly identical; main difference is that QGIS Zonal Statistics appends columns directly to the vector layer rather than creating a separate table.

## Check your understanding

- In the Zonal Statistics as Table tool, you set the Zone field to GEOID. Why is it important that GEOID — and not NAME or OBJECTID — is used as both the zone field and the join key?

- After running Zonal Statistics as Table and joining the output to your tracts, many tracts show <Null> for the MEAN field. What is the most likely cause, and how would you diagnose it?

- A colleague argues that the census tracts with the highest zonal mean LST are the most dangerous neighborhoods for heat-related illness. What is wrong with this claim? Name at least two additional data sources you would need to support or refute it.

- You reclassified LST into 5 classes using Natural Breaks. A classmate reclassified the same raster using Equal Interval. Would you expect the same tracts to appear in the “hottest” class under both methods? Why or why not?

- Why is Zonal Statistics as Table more useful for public health analysis than simply symbolizing the raw LST raster using graduated colors?