# Lab 2a: Projections Done Right

Intro GIS for Public Health · ArcGIS Pro · Day 2

## Public health question

Philadelphia’s park system is a critical health resource, but proximity depends entirely on how you measure distance. This lab asks: what share of Philadelphia children live within a 0.5-mile walk of a public park? The answer differs dramatically depending on the coordinate system used to run the buffer. One result is valid; the other is geographic nonsense. Learning to identify and fix this error before it reaches analysis is a foundational GIS skill.

## What you will learn

- Distinguish geographic coordinate systems (GCS) from projected coordinate systems (PCS)

- Read a layer’s coordinate reference system (CRS) in Layer Properties

- Interpret EPSG codes and look up unfamiliar CRS definitions at epsg.io

- Change a Map’s display coordinate system (on-the-fly reprojection)

- Permanently reproject a layer using the Project geoprocessing tool

- Run a Buffer operation in both the wrong CRS (degrees) and the correct CRS (feet) and see the difference

- Use the Measure tool to compare distances across coordinate systems

- Intersect a park buffer with census tract boundaries and compute percent of children with park access by poverty quintile

## What you will produce

- Two side-by-side maps of the same 0.5-mile park buffer — one buffer run on WGS84 data (units = degrees, wrong) and one run after reprojecting to NAD83 PA State Plane South (EPSG:2272, units = feet, correct) — displayed at the same scale so the distortion is visually obvious.

- A summary table showing the percentage of children with park access (within the 0.5-mile buffer) by census-tract poverty quintile (five groups from lowest to highest poverty rate).

- A 100-word written interpretation comparing access equity across poverty quintiles and explaining why the WGS84 buffer is invalid for this analysis.

## Data

All data are instructor-provided on the W: drive. The shorthand W:\ refers to your assigned storage drive or course folder — your instructor will confirm the exact path. Do not move or rename files.

| Layer name | Description | Source |
| --- | --- | --- |
| PPR_properties.shp | Philadelphia parks and recreation properties (polygons) | OpenDataPhilly |
| phl_tracts_acs.shp | Philadelphia census tracts with ACS child population (B09001) and poverty rate | NHGIS / Census Bureau |

Note: phl_tracts_acs.shp contains a field for total child population under 18 (CHILD_POP) and a poverty rate field (POV_RATE) already calculated from ACS Table B17001. Your instructor will confirm the exact field names for your data vintage.

## Before you begin

Prerequisites: Completion of Lab 1 (creating a project, adding data, navigating the map). No prior knowledge of projections is assumed — this lab teaches that.

Time estimate: 2.5 to 3 hours.

Have open before starting:

- ArcGIS Pro 3.x

- Windows File Explorer pointed to W:\Lab02a\

- A web browser for epsg.io lookups

## Part 1: Create the project and inspect layer coordinate systems

### Set up the project

- Open ArcGIS Pro. On the start screen, click New > Map. Name the project Lab02a_Projections, save to W:\Lab02a\, and click OK. ArcGIS Pro creates the .aprx file and a default file geodatabase.

- In the Contents pane, right-click the default map name and select Properties. Rename it WGS84_Buffer and click OK.

- On the Map tab, click Add Data. Navigate to W:\Lab02a\data\ and add PPR_properties.shp and phl_tracts_acs.shp.

[Figure: Contents pane showing PPR_properties and phl_tracts_acs layers loaded in the WGS84_Buffer map]

### Inspect each layer’s coordinate system

- In the Contents pane, right-click phl_tracts_acs and select Properties > Source. Expand the Spatial Reference section.

- You should see GCS_WGS_1984. This is a geographic coordinate system — location in degrees, not feet or meters.

- Click OK. Repeat for PPR_properties. Both layers should be in WGS84.

[Figure: Layer Properties > Source panel showing WGS84 spatial reference for phl_tracts_acs]

Note: The Map’s coordinate system and a layer’s coordinate system are different things. ArcGIS Pro reprojects layers on the fly for display, but stored coordinates do not change until you run the Project tool.

### Look up the CRS on epsg.io

- Open a web browser and go to epsg.io. Search each code and note the units:

  - 4326 — WGS 84, units: degrees (geographic)

  - 2272 — NAD83 / PA South, units: US survey foot (projected; use for Philadelphia distance work)

  - 26918 — NAD83 / UTM Zone 18N, units: metre (projected; also acceptable)

Public health note: Any analysis involving distance or area — buffers, service areas, density — requires a projected CRS with linear units (feet or meters). EPSG:4326 measures in degrees; passing it to a distance tool produces meaningless results.

## Part 2: Run a buffer in the wrong coordinate system (WGS84)

This part intentionally does the analysis wrong so you can see the consequence.

### Check the Map’s coordinate system

- In the Contents pane, right-click WGS84_Buffer and select Properties > Coordinate System. Confirm the map is set to GCS WGS 1984 (EPSG:4326). If not, search for WGS 1984, select it, and click OK.

### Run the Buffer in degrees

- On the Analysis tab, click Tools. Search Buffer and open the Buffer tool.

- Set the parameters:

| Parameter | Value |
| --- | --- |
| Input Features | PPR_properties |
| Output Feature Class | parks_buffer_WGS84 (save in your default geodatabase) |
| Distance (Linear Unit) | 0.5 |
| Unit | Degrees |
| Side Type | Full |
| Dissolve Type | All |

- Click Run.

Warning: A 0.5-mile buffer run on WGS84 data produces a buffer measured in degrees. At Philadelphia’s latitude (~40° N), 1 degree of longitude is approximately 53 miles. A buffer of 0.5 degrees is therefore roughly 26 miles wide — not half a mile. The result is geographic nonsense. You should never use degree-unit buffers for distance analysis. This step exists only to make that error visible.

- When the tool finishes, the buffer layer appears on the map. Note how enormous the buffer polygons are relative to the city boundary.

[Figure: Park buffer in WGS84 (degrees) covering a region far larger than Philadelphia — the “wrong” buffer]

### Use the Measure tool to document the distortion

- On the Map tab, in the Inquiry group, click Measure. Set units to Miles and Mode to Planar.

- Click one edge of a buffer polygon, then double-click the opposite edge. Write down the apparent diameter in miles.

Note: The result will be approximately 53 miles — confirming that “0.5 degrees” at Philadelphia’s latitude is nowhere near 0.5 miles.

- Close the Measure tool.

## Part 3: Reproject the data and run the buffer correctly

### Permanently reproject to PA State Plane South

- On the Analysis tab, click Tools to open the Geoprocessing pane. Search for Project and open the Project tool (Data Management Tools > Projections and Transformations > Project).

Note: Do not confuse Project with Define Projection. The Project tool reads an existing coordinate system and converts data to a new one, creating a new file. Define Projection only relabels a layer whose CRS information is missing or corrupted — it does not change any coordinates. Using Define Projection when you should use Project is a common and damaging mistake.

- Run the Project tool for PPR_properties:

| Parameter | Value |
| --- | --- |
| Input Dataset | PPR_properties |
| Output Dataset | parks_StatePlane (in your geodatabase) |
| Output Coordinate System | NAD 1983 StatePlane Pennsylvania South FIPS 3702 Feet (EPSG:2272) |
| Geographic Transformation | (accept ArcGIS Pro’s default suggestion) |

Navigate: globe icon > Projected Coordinate System > State Plane > NAD 1983 (US Feet) > NAD 1983 StatePlane Pennsylvania South FIPS 3702 Feet > OK.

- Click Run. parks_StatePlane appears in the Contents pane.

- Repeat for phl_tracts_acs, outputting tracts_StatePlane in EPSG:2272 with the same settings.

### Verify the reprojection

- In the Contents pane, right-click parks_StatePlane and select Properties > Source. Confirm that the Spatial Reference now shows NAD_1983_StatePlane_Pennsylvania_South_FIPS_3702_Feet.

[Figure: Layer Properties > Source showing NAD83 PA State Plane South (EPSG:2272) for parks_StatePlane]

### Set up a second Map for the correct buffer

- On the Insert tab, click New Map. Rename this map StatePlane_Buffer (right-click the map tab > Properties).

- In the Contents pane, right-click StatePlane_Buffer and select Properties > Coordinate System. Set it to NAD 1983 StatePlane Pennsylvania South FIPS 3702 Feet (EPSG:2272). Click OK.

- Add parks_StatePlane and tracts_StatePlane to this map.

### Run the Buffer in feet

- Open the Buffer tool again (Analysis tab > Tools, search Buffer).

- Set the parameters:

| Parameter | Value |
| --- | --- |
| Input Features | parks_StatePlane |
| Output Feature Class | parks_buffer_StatePlane (in your geodatabase) |
| Distance (Linear Unit) | 2640 |
| Unit | Feet |
| Side Type | Full |
| Dissolve Type | All |

2,640 feet = 0.5 miles exactly.

- Click Run.

[Figure: Park buffer in NAD83 PA State Plane South (feet) — reasonable half-mile rings around Philadelphia parks]

### Measure and compare

- With StatePlane_Buffer active, open the Measure tool (Map tab > Inquiry group). Set units to Miles, mode to Planar.

- Measure a buffer ring diameter. It should read approximately 1 mile (0.5 miles each side). Write this value alongside the WGS84 measurement from Part 2.

## Part 4: Compare the two maps side by side

- On the Insert tab, click New Layout and choose ANSI Landscape Letter.

- Use Insert > Map Frame to add the WGS84_Buffer map on the left half and StatePlane_Buffer on the right half. Resize to equal halves.

- Click each map frame and set both to the same fixed scale (approximately 1:100,000 for Philadelphia).

- Add a title, your name, a north arrow, and labels: “WGS84 Buffer (Wrong)” and “State Plane Buffer (Correct).”

- Export as Lab02a_buffer_comparison.pdf (Share > Export Layout).

[Figure: Side-by-side layout showing the enormous WGS84 buffer on the left versus the correctly-sized State Plane buffer on the right]

## Part 5: Intersect the buffer with census tracts and compute access by poverty quintile

### Intersect

- Switch to the StatePlane_Buffer map (activate it via the map tab).

- Open the Intersect tool (Analysis tab > Tools, search Intersect). Set parameters:

| Parameter | Value |
| --- | --- |
| Input Features | parks_buffer_StatePlane; tracts_StatePlane |
| Output Feature Class | buffer_tracts_intersect (in your geodatabase) |
| Join Attributes | All |

- Click Run. The output contains the portions of census tracts that fall inside the park buffer, with all original tract attributes attached.

Public health note: This intersect does not directly tell you how many children are in the buffer — it clips the tract polygons. For tracts that are only partially within the buffer, you will need to estimate the child population proportionally by area, which is an assumption known as areal interpolation. It is imperfect but standard practice when individual-level data are unavailable.

### Estimate children in the buffer by areal interpolation

- Open the attribute table of buffer_tracts_intersect (right-click layer > Attribute Table). Add two new fields: AREA_CLIP (Double) and CHILDREN_IN_BUF (Double) using Add Field.

- Right-click the AREA_CLIP field header and select Calculate Geometry. Set the property to Area and units to Square Feet. This gives the clipped area of each tract fragment.

- The instructor-provided tracts_StatePlane layer includes an AREA_FULL field for each tract’s complete area. If it is missing, run Calculate Geometry on tracts_StatePlane first, then join by GEOID to bring the value into buffer_tracts_intersect.

- Use the Field Calculator on CHILDREN_IN_BUF with the expression:

- !CHILD_POP! * (!AREA_CLIP! / !AREA_FULL!)

- This apportions children to the buffer proportional to tract area — a standard areal interpolation assumption.

### Assign poverty quintiles and summarize

- The instructor-provided tracts_StatePlane layer includes a POV_QUINTILE field (1 = lowest poverty, 5 = highest). Open the Summary Statistics tool (Analysis tab > Tools, search Summary Statistics). Set CHILDREN_IN_BUF as the statistics field (Sum), and POV_QUINTILE as the case field. Run.

- Repeat for total CHILD_POP by quintile using tracts_StatePlane directly.

- Calculate percent access for each quintile: divide CHILDREN_IN_BUF sum by CHILD_POP sum and multiply by 100. Record in the table below.

| Poverty quintile | Total children in city | Children in park buffer | % with park access |
| --- | --- | --- | --- |
| 1 (lowest poverty) |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 (highest poverty) |  |  |  |

## Deliverables

Submit the following in one PDF or as specified by your instructor:

- Side-by-side map PDF (Lab02a_buffer_comparison.pdf) showing the WGS84 buffer (wrong) and the State Plane buffer (correct) at equal scale, with labeled titles, your name, and a north arrow.

- Completed access table (the poverty quintile table from Part 5) — you may screenshot or copy it into your PDF.

- 100-word written interpretation answering: Which quintile has the highest park access rate? The lowest? What does this pattern suggest about park equity in Philadelphia? Why is the WGS84 buffer invalid for this analysis? Label any AI-assisted portions.

## Using AI in this lab

Bounded AI task — interrogating a CRS definition:

- In ArcGIS Pro, open Layer Properties > Source for any layer and copy the full Spatial Reference text block (the WKT string).

- Paste it into Claude (claude.ai) with this prompt: “Here is a coordinate reference system WKT string. Is this geographic or projected? What are its linear units? Is it appropriate for measuring distances in Philadelphia, PA? Explain why or why not.”

- Read the response critically. Does the AI correctly identify the units? Compare its answer to what you found on epsg.io. AI explains CRS text well — treat it as a scaffold, not an authority.

Guardrails:

- AI helps with explaining (what does this WKT mean?) and critiquing (is my projection choice reasonable?). It does not make analytic decisions for you.

- AI does not select the right projection for your study area — you do.

- AI does not interpret your equity findings — that requires your public health judgment.

- Never paste patient data, addresses, or any identifiable health information into a public AI tool. A WKT string is metadata and contains no PHI.

- Label any AI-assisted text in your deliverables.

## QGIS equivalent

Students using QGIS 3.x can complete the same workflow with the following equivalents:

| ArcGIS Pro operation | QGIS equivalent |
| --- | --- |
| Layer Properties > Source > Spatial Reference | Right-click layer > Properties > Information tab; or Layer CRS in the status bar |
| Map coordinate system (on-the-fly) | Project menu > Properties > CRS tab |
| Project geoprocessing tool | Vector menu > Data Management Tools > Reproject Layer (or search Reproject in the Processing Toolbox) |
| Buffer tool | Vector menu > Geoprocessing Tools > Buffer (or Processing Toolbox: native:buffer) |
| Measure tool | View menu > Measure > Measure Line |
| Intersect tool | Vector > Geoprocessing Tools > Intersection |
| Field Calculator | Right-click layer > Open Attribute Table > Field Calculator icon |

In QGIS, the project CRS defaults to the first layer loaded. Always verify the project CRS before any distance or area operation.

## Check your understanding

- A colleague sends you a shapefile of hospital locations in Philadelphia. You open it in ArcGIS Pro and it displays correctly over a basemap. Does this confirm the layer’s coordinate system is correct for distance analysis? Why or why not?

- What is the difference between the Project tool and the Define Projection tool in ArcGIS Pro? When would you use each?

- You are asked to create a 1-kilometer buffer around all FQHC clinic locations in Philadelphia. Which EPSG code would you use for the analysis — EPSG:4326, EPSG:2272, or EPSG:26918 — and why? Is there more than one acceptable answer?

- In the side-by-side map you produced, the WGS84 buffer polygon extends far beyond the city limits. Explain in plain language why a “0.5-degree” buffer at Philadelphia’s latitude produces a result that is approximately 26 miles wide.

- Your park access table shows that census tracts in the highest-poverty quintile have a lower percentage of children with park access than tracts in the lowest-poverty quintile. List two alternative explanations for this pattern that do not involve park location — and explain what additional data you would need to evaluate each explanation.